EtherChannel (also called link aggregation) is a technique used to bundle multiple physical links into a single logical link to increase bandwidth and provide redundancy. Here’s a simple example of implementing EtherChannel using LACP (Link Aggregation Control Protocol) in a network of two switches.

Network Topology

- Two switches: SW1 and SW2.

- Two physical links: SW1 Fa0/1 and Fa0/2 connected to SW2 Fa0/1 and Fa0/2.

Configuration

We’ll use LACP (active mode) for the EtherChannel configuration.

SW1 Configuration

bash

Copy code

! Enter configuration mode

SW1# configure terminal

! Select the interfaces to bundle

SW1(config)# interface range fa0/1 - 2

! Create an EtherChannel group (e.g., group 1) using LACP

SW1(config-if-range)# channel-group 1 mode active

! Configure the logical port-channel interface

SW1(config-if-range)# exit

SW1(config)# interface port-channel 1

! Set the EtherChannel to be a trunk (optional, depending on the use case)

SW1(config-if)# switchport mode trunk

SW1(config-if)# exit

! Verify configuration

SW1# show etherchannel summary

SW2 Configuration

bash

Copy code

! Enter configuration mode

SW2# configure terminal

! Select the interfaces to bundle

SW2(config)# interface range fa0/1 - 2

! Create an EtherChannel group (e.g., group 1) using LACP

SW2(config-if-range)# channel-group 1 mode active

! Configure the logical port-channel interface

SW2(config-if-range)# exit

SW2(config)# interface port-channel 1

! Set the EtherChannel to be a trunk (optional, depending on the use case)

SW2(config-if)# switchport mode trunk

SW2(config-if)# exit

! Verify configuration

SW2# show etherchannel summary

Explanation of Commands

- interface range: Select multiple interfaces to configure at once.

- channel-group 1 mode active: Adds interfaces to EtherChannel group 1 using LACP active mode. In active mode, LACP actively negotiates with the peer.

- interface port-channel 1: Configures the logical Port-Channel interface created for the EtherChannel.

- switchport mode trunk: Optional setting to configure the EtherChannel as a trunk link.

Verification

- Check EtherChannel Status

bash

Copy code

SW1# show etherchannel summary

  - Look for Group and Port-Channel details.

  - Ensure the Ports are in P (port-channel) or S (standby) state.

- Check Trunking (if applicable)

bash

Copy code

SW1# show interfaces trunk

Troubleshooting

- Ensure identical configurations on both sides (e.g., speed, duplex, VLAN settings).

- Use show etherchannel commands to diagnose issues:

  - show etherchannel detail

  - show spanning-tree (to confirm spanning-tree sees the link correctly).

Let me know if you’d like to extend this setup with VLANs or spanning-tree optimization!