Reset all devices in CLI

Router

int g0/0/0

no ip add

shut

no ip nat

int g0/0/1

no ip add

shut

no ip nat

int g0/1/0

no ip add

shut

no ip nat

For Laptop to internet Rack 6D

!To internet

Hostname R2

int g0/0/1

no shut

ip address 172.17.9.241 255.255.255.252

ip nat outside

! Personal Laptop for internet usage

interface g 0/1/0

no shut

ip nat inside

ip add 10.10.10.1 255.255.255.0

! Route to ISP router

ip route 0.0.0.0 0.0.0.0 172.17.9.242

! Dynamic NAT Pool out to internet

ip nat pool NAT-POOL1 203.149.211.224 203.149.211.230 netmask 255.255.255.248

access-list 1 permit 10.10.10.0 0.0.0.255

ip nat inside source list 1 pool NAT-POOL1

Router 1

!configure router 1

R1(config)#int g0/0/1

R1(config-if)#ip add 192.168.30.81 255.255.255.248

R1(config-if)#no sh

R1(config-if)#exit

ip dhcp pool VLAN-10

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

domain-name google.com

lease 2

ip dhcp pool VLAN-20

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

domain-name google.com

lease 2

ip dhcp excluded-address 192.168.10.1 192.168.10.9

ip dhcp excluded-address 192.168.20.1 192.168.20.9

ip route 0.0.0.0 0.0.0.0 192.168.30.82

DSW1

!set up name for vlan

en

Conf t

Hostname DSW2

Vlan 10

name Ops

vlan 20

name Engg

!Comms to R1

Vlan 30

name Svr

!setup intervlan config

int vlan 10

!Set DSW1 .2 // DSW2 .3

ip add 192.168.10.2 255.255.255.0

no shut

ip helper-address 192.168.30.81

int vlan 20

ip add 192.168.20.2 255.255.255.0

ip helper-address 192.168.30.81

no shut

int vlan 30

!DSW1 .83 // DSW2 .84

ip add 192.168.30.83 255.255.255.248

no shut

!For access to vlan30 subnet

Int g1/0/13

Switchport mode access

Switchport access vlan 30

No sh

! Allow vlan 10,20,30

Int g1/0/13

Switchport mode trunk

Switchport trunk allowed vlan 10,20,30

No sh

exit

!For trunking to SW1

!g1/0/1 for DSW1

!g1/0/24 for DSW2

Int ran g1/0/12,g1/0/1

No shut

Switchport mode trunk

Switchport trunk allowed vlan 10,20

End

!For routing

Conf t

ip routing

!For HSRP

!Priority for active L3 switch set to 150

!Priority for standby L3 switch set to 50

en

End

Conf t

!For Vlan 10

Int vlan 10

standby ver 2
standby 10 ip 192.168.10.1

standby 10 priority 150
standby 10 preempt

!For Vlan 20

Int vlan 20

standby ver 2
standby 20 ip 192.168.20.1
standby 20 priority 150
standby 20 preempt

!For Vlan 30

Int vlan 30

standby ver 2
standby 30 ip 192.168.30.82
standby 30 priority 150
standby 30 preempt

! For ASA internet out

ip route 0.0.0.0 0.0.0.0 192.168.10.5

ip route 0.0.0.0 0.0.0.0 192.168.20.5

DSW 2

!set up name for vlan

en

Conf t

Hostname DSW2

!

Vlan 10

name Ops

vlan 20

name Engg

!Comms to R1

Vlan 30

name Svr

!setup intervlan config

int vlan 10

!Set DSW1 .2 // DSW2 .3

ip add 192.168.10.3 255.255.255.0

no shut

ip helper-address 192.168.30.81

int vlan 20

ip add 192.168.20.3 255.255.255.0

ip helper-address 192.168.30.81

no shut

int vlan 30

!DSW1 .83 // DSW2 .84

ip add 192.168.30.84 255.255.255.248

no shut

! Allow vlan 10,20,30

Int g1/0/13

Switchport mode trunk

Switchport trunk allowed vlan 10,20,30

No sh

exit

!For trunking to SW1

Int ran g1/0/12,g1/0/24

No shut

Switchport mode trunk

Switchport trunk allowed vlan 10,20

Exit

!For routing

end

Conf t

ip routing

!For HSRP

!Priority for active L3 switch set to 150

!Priority for standby L3 switch set to 50

en

End

Conf t

!For Vlan 10

Int vlan 10

standby ver 2
standby 10 ip 192.168.10.1
standby 10 priority 50
standby 10 preempt

!For Vlan 20

Int vlan 20

standby ver 2
standby 20 ip 192.168.20.1
standby 20 priority 50
standby 20 preempt

!For Vlan 30

Int vlan 30

standby ver 2
standby 30 ip 192.168.30.82
standby 30 priority 50
standby 30 preempt

! For ASA internet out

ip route 0.0.0.0 0.0.0.0 192.168.10.5

ip route 0.0.0.0 0.0.0.0 192.168.20.5

SW2

En

Conf t

Hostname SW2

Int range g1/0/1

No shut

Switchport mode access

switchport access vlan 30

! Allow vlan 10,20,30 to ASA

Int ran g1/0/12-13, g1/0/2

Switchport mode trunk

Switchport trunk allowed vlan 10,20,30

No sh

exit

exit

SW1

En

Conf t

Hostname SW2

!Trunking

Int range g1/0/1,g1/0/24

Switchport mode trunk

Switchport trunk allowed vlan 10,20

!Access to vlan 10

Int range g1/0/2-11

Switchport mode access

Switchport access vlan 10

!Access to vlan 20

Int range g1/0/13-23

Switchport mode access

Switchport access vlan 20

Black hole on unused port

Vlan 999

Name Blackhole

Int range g1/1/1-4,G1/0/2-23

Switchport mode access

Switchport access vlan 999

Shutdown

Exit

!All trunk interface disallow vlan 999

Int ran g1/0/1,g1/0/24

Switchport mode trunk

Switchport trunk allowed vlan except 999

Native Vlan  on Trunk

!All trunk interface to set same native vlan

Int ran g1/0/1,g1/0/24

Switchport trunk native vlan 616

Defence DHCP on Vlan 10 only

ip dhcp snooping

int ran g1/0/1,g1/0/24

ip dhcp snooping trust

int range g1/0/2-11

ip dhcp snooping limit rate 6

exit

ip dhcp snooping vlan 10

ip dhcp snooping verify mac-address

no ip dhcp snooping information option

ARP Poisoning on vlan 20 only

ip dhcp snooping

ip dhcp snooping vlan 20

ip arp inspection vlan 20

int ran g1/0/1,g1/0/24

ip dhcp snooping trust

ip arp inspection trust

ip arp inspection validate dst-mac ip src-mac

no ip dhcp snooping information option

Portfast

!on access port

Spanning-tree portfast

Slowloris attack

FW1(config)# access-list serverA extended permit tcp any host 192.168.50.209 eq 80

FW1(config)# class-map http_serverA

FW1(config-cmap)# match access-list serverA

FW1(config-cmap)# exit

FW1(config)# policy-map policy_serverA

FW1(config-pmap)# class http_serverA

FW1(config-pmap-c)# set connection conn-max 100

FW1(config-pmap-c)# set connection per-client-max 10

FW1(config-pmap-c)# exit

FW1(config-pmap)# exit

FW1(config)# service-policy  policy_serverA interface outside

Anti-spoofing entries

!Only allow this internal network to server internet

R2(config)#access-list 110 permit ip 192.168.40.100 0.0.0.7 any

R2(config)#access-list 110 deny ip any any

!Only allow this defined network from entering inside network

R2(config)#access-list 120 permit ip 172.17.9.244 0.0.0.3 any

R2(config)#access-list 120 permit ip 203.149.211.232 0.0.0.7 any

R2(config)#access-list 120 deny ip any any

Configure traffic to internet NAT on router  (Rack 7D)

End

Conf t

hostname R2

! From router to outside internet

int g0/0/1

no shut

ip address 172.17.9.241	255.255.255.252

ip nat outside

! From router to Inside

int g0/0/0

no sh

ip add 192.168.40.102 255.255.255.0

ip nat inside

! Route outwards to internet, DMZ, Inside

ip route 0.0.0.0 0.0.0.0 172.17.9.242

ip route 192.168.50.200 255.255.255.248 192.168.40.101

ip route 192.168.30.80 255.255.255.248 192.168.40.101

! NAT Translation from Private to Public

ip nat pool NAT-POOL1 203.149.211.224 203.149.211.229 netmask 255.255.255.248

! Access-list to allow network to use Public IP

access-list 1 permit 192.168.40.100  0.0.0.7

ip nat inside source list 1 pool NAT-POOL1

Config ASA (Firewall)

hostname FW1

int e1/1

no shut

no switchport

nameif OUTSIDE

security-level 0

ip add 192.168.40.101 255.255.255.252

int e1/3

no shut

no switchport

nameif DMZ

security-level 50

ip add 192.168.50

.201 255.255.255.248

int e1/2

no shut

no switchport

no nameif

no security-level

no ip add

! DMZ setting

! DMZ routing

route outside  0 0 192.168.40.102

object network DMZ-outside

subnet 192.168.50.200 255.255.255.248

nat (DMZ,outside) dynamic interface

! ACL for DMZ

access-list OUTACCESS-DMZ permit tcp 192.168.50.200 255.255.255.248 any eq 8080

access-list OUTACCESS-DMZ permit tcp 192.168.50.200 255.255.255.248 any eq 80

access-list OUTACCESS-DMZ permit icmp 192.168.50.200 255.255.255.248 any echo

access-group OUTACCESS-DMZ in interface outside

! MPF

class-map inspection_default

match default-inspection-traffic

exit

policy-map global_policy

class inspection_default

inspect icmp

exit

exit

service-policy global_policy global

! Inside Vlan10,20,30 setting

! Subinterface

Int e1/2

No shut

int e1/2.10

no shut

vlan10

nameif inside-vlan10

security-level 100

ip add 192.168.10.5 255.255.255.0

int e1/2.20

no shut

vlan20

nameif inside-vlan20

security-level 100

ip add 192.168.20.5 255.255.255.0

!allow same security access

same-security-traffic permit intra-interface

! Inside routing

route inside  192.168.10.0 255.255.255.0 192.168.10.1

route inside  192.168.20.0 255.255.255.0 192.168.20.1

object network Vlan10-outside

subnet 192.168.10.0 255.255.255.0

nat (inside,outside) dynamic interface

object network Vlan20-outside

subnet 192.168.20.0 255.255.255.0

nat (inside,outside) dynamic interface

object network Vlan30-outside

subnet 192.168.30.80 255.255.255.248

nat (inside,outside) dynamic interface

access-list INSIDE-OUT permit tcp any any eq 80

!ACL for inside Vlan30

access-list INSIDE-OUT permit tcp 192.168.30.80 255.255.255.248 any eq 8080

access-list INSIDE-OUT permit tcp 192.168.30.80 255.255.255.248 any eq 80

access-list INSIDE-OUT permit icmp 192.168.30.80 255.255.255.248 any echo

!ACL for inside Vlan10

access-list INSIDE-OUT permit tcp 192.168.10.0 255.255.255.0 any eq 8080

access-list INSIDE-OUT permit tcp 192.168.10.0 255.255.255.0 any eq 80

access-list INSIDE-OUT permit icmp 192.168.10.0 255.255.255.0 any echo

!ACL for inside Vlan20

access-list INSIDE-OUT permit tcp 192.168.20.0 255.255.255.0 any eq 8080

access-list INSIDE-OUT permit tcp 192.168.20.0 255.255.255.0 any eq 80

access-list INSIDE-OUT permit icmp 192.168.20.0 255.255.255.0 any echo

access-group INSIDE-OUT in interface outside

Configure DHCP

Router(config)#host R1

R1(config)#int g0/0/0

R1(config-if)#no sh

R1(config-if)#ip add 192.168.30.81 255.255.255.248

R1(config-if)#exit

R1(config)#int g0/0/0.10

R1(config-subif)#encapsulation dot1Q 10

R1(config-subif)#ip add 192.168.10.1 255.255.255.0

R1(config-subif)#ip helper-address 192.168.10.1

R1(config-subif)#exit

R1(config)#int g0/0/0.20

R1(config-subif)#encapsulation dot1Q 20

R1(config-subif)#ip add 192.168.20.1 255.255.255.0

R1(config-subif)#ip helper-address 192.168.20.1

R1(config-subif)#exit

R1(config)#ip dhcp pool Vlan-10

R1(dhcp-config)#network 192.168.10.0 255.255.255.0

R1(dhcp-config)#dns-server 8.8.8.8

R1(dhcp-config)#default-router 192.168.10.1

R1(config)#ip dhcp pool Vlan-20

R1(dhcp-config)#network 192.168.20.0 255.255.255.0

R1(dhcp-config)#dns-server 8.8.8.8

R1(dhcp-config)#default-router 192.168.20.1

R1(config)#ip dhcp excluded-address 192.168.10.1 192.168.10.100

R1(config)#ip dhcp excluded-address 192.168.20.1 192.168.20.100

Switch(config)#hostname SW2

SW2(config)# int range g1/0/1,g1/0/12-13

SW2(config-if-range)#switchport mode trunk

SW2(config-if-range)#switchport trunk allowed vlan 10,20

Switch(config)#hostname DSW1

DSW1(config)# int ran g1/0/12-13,g1/0/1

DSW1 (config-if-range)#switchport mode trunk

DSW1 (config-if-range)#switchport trunk allowed vlan 10,20

DSW1(config)#vlan 10

DSW1(config-vlan)#name VLAN10

DSW1(config-vlan)#vlan 20

DSW1(config-vlan)#name VLAN20

DSW1(config)#Int vlan 10

DSW1(config-if)#Ip helper add 192.168. 30.81

DSW1(config)#Int vlan 20

DSW1(config-if)#Ip helper add 192.168. 30.81

Deviant to test L3 intervlan routing

Switch(config)#hostname DSW1

DSW1(config)#int g1/0/13

DSW1(config-if)#ip add 192.168.30.82 255.255.255.248

DSW1(config)#interface vlan 10

DSW1(config-if)#ip add 192.168.10.1 255.255.255.0

DSW1(config)#interface vlan 20

DSW1(config-if)#ip add 192.168.20.1 255.255.255.0

DSW1(config)#interface vlan 99

DSW1(config-if)#ip add 192.168.99.1 255.255.255.0

DSW1(config)#int g1/0/1

DSW1 (config-if)#switchport mode trunk

DSW1 (config-if)#switchport truck native vlan 99

DSW1(config)#ip routing

SW2#int g1/0/1

SW2(config-if)#switchport mode trunk

SW2(config-if)#switchport truck native vlan 99

Switch(config)#hostname DSW2

DSW2(config)# int ran g1/0/12-13,g1/0/24

DSW2 (config-if-range)#switchport mode trunk

DSW2 (config-if-range)#switchport trunk allowed vlan 10,20

DSW2(config)#vlan 10

DSW2(config-vlan)#name VLAN10

DSW2(config-vlan)#vlan 20

DSW2(config-vlan)#name VLAN20

DSW2(config)#Int vlan 10

DSW2(config-if)#Ip helper add 192.168. 30.81

DSW2(config)#Int vlan 20

DSW2(config-if)#Ip helper add 192.168. 30.81

Switch(config)#hostname SW1

SW1(config)#int ran f0/1,f0/24

SW1(config-if-range)#switchport mode trunk

SW1(config-if-range)#switchport trunk allowed vlan 10,20

SW1(config)#vlan 10

SW1(config-vlan)#name VLAN10

SW1(config-vlan)#exit

SW1(config)#int vlan 10

SW1(config)#no shut

SW1(config)#vlan 20

SW1(config-vlan)#name VLAN20

SW1(config-vlan)#exit

SW1(config)#int vlan 20

SW1(config)#no shut

SW1 (config)# int ran g1/0/2-11

SW1 (config-if-range)#switchport mode access

SW1 (config-if-range)#switchport access vlan 10

SW1 (config)# int ran g1/0/13-23

SW1 (config-if-range)#switchport mode access

SW1 (config-if-range)#switchport access vlan 20

Configure HSRP

DSW1(config)# interface Vlan10

DSW1(config-if)#ip add 192.168.10.2 255.255.255.0

DSW1(config-if)#standby version 2

DSW1(config-if)#standby 10 ip 192.168.10.1

DSW1(config-if)#standby 10 priority 150

DSW1(config-if)#standby 10 preempt

DSW1(config)# interface Vlan20

DSW1(config-if)#ip add 192.168.20.2 255.255.255.0

DSW1(config-if)#standby version 2

DSW1(config-if)#standby 10 ip 192.168.20.1

DSW1(config-if)#standby 10 priority 150

DSW1(config-if)#standby 10 preempt

DSW2(config)# interface Vlan10

DSW2(config-if)#ip add 192.168.10.2 255.255.255.0

DSW2(config-if)#standby version 2

DSW2(config-if)#standby 10 ip 192.168.10.1

DSW2(config-if)#standby 10 priority 150

DSW2(config-if)#standby 10 preempt

DSW2(config)# interface Vlan20

DSW2(config-if)#ip add 192.168.20.2 255.255.255.0

DSW2(config-if)#standby version 2

DSW2(config-if)#standby 10 ip 192.168.20.1

DSW2(config-if)#standby 10 priority 150

DSW2(config-if)#standby 10 preempt

Defence DHCP on Vlan 10 only

SW1(config)#ip dhcp snooping

SW1(config)#int ran f0/1,f0/24

SW1(config-if-range)#ip dhcp snooping trust

SW1(config)#int range f0/2-11

SW1(config-if-range)#ip dhcp snooping limit rate 6

SW1(config-if-range)#exit

SW1(config)#ip dhcp snooping vlan 10

SW1(config)#ip dhcp snooping verify mac-address

SW1(config)#no ip dhcp snooping information option

ARP Poisoning on vlan 20 only

SW1(config)#ip dhcp snooping

SW1(config)#ip dhcp snooping vlan 20

SW1(config)#ip arp inspection vlan 20

SW1(config)#int ran f0/1,f0/24

SW1(config-if-range)#ip dhcp snooping trust

SW1(config-if-range)#ip arp inspection trust

SW1(config)#ip arp inspection validate dst-mac ip src-mac

SW1(config)#no ip dhcp snooping information option

Configure NAT  (Rack 7E)

Router(config)#hostname R2

//To internet

R2(config)# int g0/0/1

R2(config-if)# no shut

R2(config-if)# ip address 172.17.9.213	255.255.255.252

R2(config-if)# ip nat outside

//To Inside

R2 (config-if)#int g0/0/0

R2 (config-if)#no sh

R2 (config-if)#ip add 192.168.40.1 255.255.255.0

R2(config-if)#ip nat inside

//Personal Laptop for internet usage

R2(config)# interface g 0/1/0

R2(config-if)# ip nat inside

R2(config-if)#ip add 10.10.10.1 255.255.255.0

R2(config)#ip route 0.0.0.0 0.0.0.0 172.17.9.246

R2(config)#ip route 192.168.50.208 255.255.255.248 192.168.40.101

R2(config)# ip nat pool NAT-POOL1 203.149.211.168 203.149.211.173 netmask 255.255.255.248

R2(config)#access-list 1 permit 10.10.10.0 0.0.0.255

R2(config)#access-list 1 permit 192.168.40.100 0.0.0.7

R2(config)# ip nat inside source list 1 pool NAT-POOL1

Config ASA (Firewall)

ciscoasa (config)#hostname FW1

FW1(config)# int e1/1

FW1(config-if)# no switchport

FW1(config-if)# nameif OUTSIDE

FW1(config-if)# security-level 0

FW1(config-if)# ip add 192.168.40.10 255.255.255.0

FW1(config)# int e1/3

FW1(config-if)# no switchport

FW1(config-if)# nameif DMZ

FW1(config-if)# security-level 50

FW1(config-if)# ip add 192.168.50.210 255.255.255.248

FW1(config)# route outside  0 0 192.168.40.1

FW1(config)# object network DMZ-outside

FW1(config-network-object)# subnet 192.168.50.208 255.255.255.248

FW1(config-network-object)# nat (DMZ,outside) dynamic interface

FW1(config)# access-list OUTACCESS-DMZ permit tcp 192.168.50.208 255.255.255.248 any eq 8080

FW1(config)# access-list OUTACCESS-DMZ permit tcp 192.168.50.208 255.255.255.248 any eq 80

FW1(config)# access-list OUTACCESS-DMZ permit icmp 192.168.50.208 255.255.255.248 any echo

FW1(config)# access-group OUTACCESS-DMZ in interface outside

FW1(config)# class-map inspection_default

FW1(config-cmap)# match default-inspection-traffic

FW1(config-cmap)# exit

FW1(config)# policy-map global_policy

FW1(config-pmap)# class inspection_default

FW1(config-pmap-c)# inspect icmp

FW1(config-pmap-c)# exit

FW1(config-pmap)# exit

FW1(config)# service-policy global_policy global

Defence

Slowloris attack

FW1(config)# access-list serverA extended permit tcp any host 192.168.50.209 eq 80

FW1(config)# class-map http_serverA

FW1(config-cmap)# match access-list serverA

FW1(config-cmap)# exit

FW1(config)# policy-map policy_serverA

FW1(config-pmap)# class http_serverA

FW1(config-pmap-c)# set connection conn-max 100

FW1(config-pmap-c)# set connection per-client-max 10

FW1(config-pmap-c)# exit

FW1(config-pmap)# exit

FW1(config)# service-policy  policy_serverA interface outside

Anti-spoofing entries

!Only allow this internal network to server internet

R2(config)#access-list 110 permit ip 192.168.40.100 0.0.0.7 any

R2(config)#access-list 110 deny ip any any

!Only allow this defined network from entering inside network

R2(config)#access-list 120 permit ip 172.17.9.244 0.0.0.3 any

R2(config)#access-list 120 permit ip 203.149.211.232 0.0.0.7 any

R2(config)#access-list 120 deny ip any any

Router DHCP

ip dhcp excluded-address 192.168.10.1 192.168.10.100

ip dhcp excluded-address 192.168.20.1 192.168.20.100

ip dhcp excluded-address 192.168.30.81

!

ip dhcp pool vlan_10

network 192.168.10.0 255.255.255.0

dns-server 8.8.8.8

default-router 192.168.10.1

domain-name google.com

!

ip dhcp pool vlan_20

network 192.168.20.0 255.255.255.0

dns-server 8.8.8.8

default-router 192.168.20.1

domain-name google.com

!

ip dhcp pool helperaddress

network 192.168.30.80 255.255.255.248

interface GigabitEthernet0/0/0

no ip address

negotiation auto

!

interface GigabitEthernet0/0/1

ip dhcp client client-id ascii FGL2802L0MG

ip address 192.168.30.81 255.255.255.248

negotiation auto

!

interface GigabitEthernet0/0/1.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

!

interface GigabitEthernet0/0/1.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

DSW1

ip routing

interface GigabitEthernet1/0/1

switchport trunk allowed vlan 10,20

switchport mode trunk

!

interface GigabitEthernet1/0/12

switchport trunk allowed vlan 10,20

switchport mode trunk

!

interface GigabitEthernet1/0/13

switchport trunk allowed vlan 10,20

switchport mode trunk

!

interface Vlan10

ip address 192.168.10.2 255.255.255.0

ip helper-address 192.168.30.81

standby version 2

standby 10 ip 192.168.10.1

standby 10 priority 110

standby 10 preempt

!

interface Vlan20

ip address 192.168.20.2 255.255.255.0

ip helper-address 192.168.30.81

standby version 2

standby 20 ip 192.168.20.1

standby 20 priority 110

standby 20 preempt

!

DSW2

interface GigabitEthernet1/0/12

switchport trunk allowed vlan 10,20

switchport mode trunk

!

interface GigabitEthernet1/0/13

switchport trunk allowed vlan 10,20

switchport mode trunk

interface GigabitEthernet1/0/24

switchport trunk allowed vlan 10,20

switchport mode trunk

!

!

interface Vlan10

ip address 192.168.10.3 255.255.255.0

ip helper-address 192.168.30.81

standby version 2

standby 10 ip 192.168.10.1

standby 10 preempt

!

interface Vlan20

ip address 192.168.20.3 255.255.255.0

ip helper-address 192.168.30.81

standby version 2

standby 20 ip 192.168.20.1

standby 20 preempt