Question 1

Updated VLAN Subnet Allocation

To avoid overlap, I'll ensure the VLAN IPs only use a portion of the subnet, leaving room for network devices (routers, firewalls, switches, etc.) to have unique IPs.

| VLAN Number | VLAN Name | Subnet | Subnet Mask | Host Range | Purpose |
| --- | --- | --- | --- | --- | --- |
| 10 | Lab1 | 192.168.1.0/26 | 255.255.255.192 | 192.168.1.1-192.168.1.62 | Lab1 devices (14 computers) |
| 20 | Lab2 | 192.168.1.64/26 | 255.255.255.192 | 192.168.1.65-192.168.1.126 | Lab2 devices (14 computers) |
| 30 | Lab3 | 192.168.1.128/27 | 255.255.255.224 | 192.168.1.129-192.168.1.158 | Lab3 devices (14 computers) |
| 40 | Lab4 | 192.168.1.160/27 | 255.255.255.224 | 192.168.1.161-192.168.1.190 | Lab4 devices (14 computers) |
| 50 | Staff | 192.168.1.192/28 | 255.255.255.240 | 192.168.1.193-192.168.1.206 | Staff devices (principal, instructors, admin) |
| 60 | IT Support | 192.168.1.208/28 | 255.255.255.240 | 192.168.1.209-192.168.1.222 | IT support devices (Admin Support & Engineers) |
| 99 | Management | 192.168.1.224/28 | 255.255.255.240 | 192.168.1.225-192.168.1.238 | Network device management |

Revised Device IP Allocation

We’ll assign IP addresses for firewalls, routers, Layer 3 switches, and Layer 2 switches outside the VLAN host ranges.

| Device | Hostname | Interface/Port | IP Address | Purpose |
| --- | --- | --- | --- | --- |
| Firewall 1 | FW1 | Gi0/0 (LAN) | 192.168.1.225 | Connected to R1 LAN |
|  |  | Gi0/1 (ISP1) | 203.149.210.33 | Public IP from ISP1 |
| Firewall 2 | FW2 | Gi0/0 (LAN) | 192.168.1.226 | Connected to R2 LAN |
|  |  | Gi0/1 (ISP2) | 129.126.142.57 | Public IP from ISP2 |
| Router 1 | R1 | Gi0/0 | 192.168.1.227 | Connected to FW1 LAN |
|  |  | Gi0/1 | 192.168.1.229 | Connected to DSW1 |
| Router 2 | R2 | Gi0/0 | 192.168.1.228 | Connected to FW2 LAN |
|  |  | Gi0/1 | 192.168.1.230 | Connected to DSW2 |
| L3 Switch 1 | DSW1 | VLAN 99 | 192.168.1.232 | Management VLAN HSRP |
|  |  | Gi0/4 | 192.168.1.234 | Connected to R1 |
|  |  | Gi0/5 | Trunk | Connected to DSW2 |
| L3 Switch 2 | DSW2 | VLAN 99 | 192.168.1.233 | Management VLAN HSRP |
|  |  | Gi0/4 | 192.168.1.235 | Connected to R2 |
|  |  | Gi0/5 | Trunk | Connected to DSW1 |

Revised VLAN Gateway Assignments

Each VLAN gateway will be configured on the Layer 3 switches.

| VLAN Number | Gateway IP Address | Device |
| --- | --- | --- |
| 10 | 192.168.1.1 | DSW1, DSW2 |
| 20 | 192.168.1.65 | DSW1, DSW2 |
| 30 | 192.168.1.129 | DSW1, DSW2 |
| 40 | 192.168.1.161 | DSW1, DSW2 |
| 50 | 192.168.1.193 | DSW1, DSW2 |
| 60 | 192.168.1.209 | DSW1, DSW2 |
| 99 | 192.168.1.231 | DSW1, DSW2 |

Additional Tables

Table: Public IP Assignment

| Public Address Block | Assigned To | IP Address | Purpose |
| --- | --- | --- | --- |
| 203.149.210.32/29 | FW1 WAN Interface | 203.149.210.33 | Firewall (ISP1 WAN) |
|  | NAT Pool | 203.149.210.34-203.149.210.38 | NAT for internal traffic |
| 129.126.142.56/29 | FW2 WAN Interface | 129.126.142.57 | Firewall (ISP2 WAN) |
|  | NAT Pool | 129.126.142.58-129.126.142.62 | NAT for internal traffic |

Table: Redundant Links

| Device 1 | Interface | Device 2 | Interface |
| --- | --- | --- | --- |
| DSW1 | Gi0/5 | DSW2 | Gi0/5 |
| SW1 | Gi0/1 | DSW1 | Gi0/7 |
| SW2 | Gi0/1 | DSW1 | Gi0/8 |
| SW3 | Gi0/1 | DSW2 | Gi0/7 |
| SW4 | Gi0/1 | DSW2 | Gi0/8 |

This updated design resolves overlapping issues by ensuring:

- Device IPs are allocated outside the VLAN host ranges.

- VLAN gateways have unique IPs within their subnet.

- Public IPs for WAN connections and NAT pools remain efficient.