Question 1

With the promotion of lifelong learning and availability of SkillsFuture credit, a startup company is setting up an IT training school with the following facilities and staff:

- 14 computers each in 4 computer labs which can be re- configured easily by applying suitable ACLs only:

  - To prevent learners from disrupting other classes, each lab network may be isolated from other labs when required,

  - To support larger classes, a few lab networks may be able to inter-communicate but not with the rest,

  - If necessary, each lab network may be allowed or denied access to the Internet.

- 1 computer each for school's staff comprising of 1 x school principal, 6 x instructors, 4 x admin support, 3 x finance officers, and 2 x network engineers.

For Internet access, the company has subscribed to 2 x ISPs to achieve better reliability.

ISP1 has allocated the IP address 172.17.9.17/30 for your network device to connect to it. In addition, for Internet access, the ISP1 has allocated the public IP address block 203.149.210.32/29.

Similarly, ISP2 has allocated the IP address 172.17.10.29/30 for your network device to connect to it. in addition, for Internet access, the ISP2 has allocated the public IP address block 129.126.142.56/29.

For internal enterprise network, the school's network engineers have decided to use the IP address block 192.168.1.0/24.

Based on the network devices available on a rack in the lab, design a secure scalable enterprise network with redundancy to support the branch school. Present your design clearly in a logical network diagram indicating clearly:

- network devices with given hostnames of your choice,

- connection links labelled with exact interface/port numbers of the devices,

- VLAN numbers, subnet address blocks and subnet masks,

- and IP addresses assigned to the interfaces of the network devices if applicable (excluding IP addresses assigned to user PCs/laptops).